Advanced Lane Finding on the Road

Goals

- Compute the camera calibration matrix and distortion coefficients given a set of chessboard images.

- Apply a distortion correction to raw images.

- Use color transforms, gradients, etc., to create a threshold binary image.

- Apply a perspective transform to rectify binary image ("birds-eye view").

- Detect lane pixels and fit to find the lane boundary.

- Determine the curvature of the lane and vehicle position with respect to center.

- Warp the detected lane boundaries back onto the original image.

- Output visual display of the lane boundaries and numerical estimation of lane curvature and vehicle position.

### Camera Calibration

I start by preparing "object points", which will be the (x, y, z) coordinates of the chessboard corners in the world. Here I am assuming the chessboard is fixed on the (x, y) plane at z=0, such that the object points are the same for each calibration image. Thus, objp is just a replicated array of coordinates, and obj_points will be appended with a copy of it every time I successfully detect all chessboard corners in a test image. image_points will be appended with the (x, y) pixel position of each of the corners in the image plane with each successful chessboard detection.

I then used the output obj_points and image _points to compute the camera calibration and distortion coefficients using the cv2.calibrateCamera() function. I applied this distortion correction to the test image using the cv2.undistort() function and obtained this result:

### Pipeline (single images)

### Pipeline consists of six steps and they are as follows

- Undistort the image using the values you calculated in camera calibration phase.

- Thresholding

  - Convert to HLS color space and separate the S channel.

  - Convert to gray scale

  - Find gradients in x direction.

  - Threshold x gradient and color channel.

  - Combine the two binary thresholds.

- Extract the region of interest.

  - Usually the road appears below in the image and does not appear above.

  - Also, it appears towards the center of the image.

  - So, extracting relevant portion of image helps us to eliminate all the edges in rest of regions in the image.

- Apply perspective transform to get bird eye view

- Find lane pixels

  - Take histogram of bottom half of image.

  - Find peaks in left and right halves of the histogram.

  - Choose the hyper parameters for number of sliding windows, width of the sliding window, minimum no of pixels found to re-center window.

  - Identify x and y positions of all non-zero pixels in the image.

  - Step through windows one by one.

    - Identify window boundaries in x and y

    - Draw the windows on the visualization image

    - Identify the nonzero pixels in x and y within the window

    - Append these indices to the lists

    - If you found > minpix pixels, recenter next window on their mean position

    - Concatenate the arrays of indices

    - Extract left and right line pixel positions

- Fit Polynomial

- Measure curvature of pixels

- Highlight the area between the lanes

- Display the highlighted area on top of image

### Link to output on video

### https://drive.google.com/file/d/1MyQarhEFBFu9vWtCbYfiDZ9lmlgmqgV_/view?usp=sharing

### Discussion

- Had hard time finding the right src and dst points for perspective transform

### Improvements

- I see that the algorithm shakes a liitle bit when the lane color changes and could be improved in that a bit